Modem Initialization Strings

1. Standard 2400 bps or 1200 bps Hayes Compatibles (NON-MNP):

If your modem supports the extended "&" command set, then try:

ATX4Q0E0V0&C1&D2S2=251S7=40

X4     Enables extended result codes

Q0     Modem returns Result Codes

E0     Modem does not echo commands back to the computer

V0     Modem returns Numeric Result Codes

&C1    Modem asserts Carrier Detect based on connection status

&D2    On DTR high-to-low transition, Modem Hangs Up.

S2=251 Disable Modem Escape Sequence

S7=40  Time Modem waits for a carrier when initiating a new connection

If your modem does not support the & command set, try using:

ATX4Q0E0V0S2=251S7=40

Modems which do not use the & command set usually have DIP switches of some type

to set their mode of operation. Verify that your modem is set to:

a) Assert CD (carrier detect) only when conencted to a remote modem

b) Hang Up on drop of DTR (data terminal ready)

2. US Robotics Courier HST

In numerous tests, the best results were obtained using:

ATX7Q0E0V0&H1&R2&B1H0

X7     Enables HST extended result codes

&H1    Turns on hardware handshaking for output from DTE (CTS line) *

&R2    Turns on hardware handshaking for input to DTE    (RTS line) *

&B1    Fixes the DTE to DCE rate **

*You must also enable the hardware handshaking in VCONFIG Ports Configuration.

**You must also enable FIXED baud rate in VCONFIG Ports Configuration.